COMPONENTS OF A REHABILITATION WORK WRITE-UP:

One of the most difficult aspects of housing rehabilitation is the development of a complete, thorough, and unambiguous Work Write-Up specific to the structure to be rehabilitated. Learning the system by which a house is assembled and constructed is a time consuming endeavor, moreover, it takes an understanding of how systems “fit together” so as to assess the extent of the rehabilitation required. For example: a qualified Rehab Specialist encountering a leaking window, will understand that a window unit is set into, and made part of, the external frame work of the house. Therefore he realizes that the leaking window can potentially impact the integrity of the adjacent framing members surrounding the window frame. A good Work Write-Up will address not only the window but will call for an inspection of the supporting structural members around the window unit itself.

The Georgia Department of Community Affairs has developed and made available to all grant recipients and/or their contracted administrators a suggested format by which you can develop both a comprehensive and detailed Work Write-Up (WWU) and a Request for Bid (RFB). Whereas the process used to create a viable and detailed WWU is relatively simple, the need to thoroughly inspect and document each of the tasks to be accomplished still requires a complete inspection of the structure itself. Knowledge of how to perform an authoritative inspection remains a vital component to the process.

As mentioned in the Introduction, DCA has made available a set of Specifications that may be used by all CDBG and CHIP grant administrators. This set of specifications covers almost all tasks that will normally be encountered by the Rehab Specialist. Each Specification contains four main components:

- A detailed description of the individual task to be completed

- A “ Spec” number

- A “Unit of Measure” that is appropriate to the specific task

- A “Unit Cost” associated with that individual Specification and Unit of Measure

You will also find other important information on the Specifications Data Sheet:

- A “Trade ID” number: a coded reference identifying each major trade or specialty. (The importance of this Trade ID number will be evident.)

- A “Mod Date”, the date of the last Unit Cost sampling

- A “Spec Title”, a brief title of the work to be performed.

We will discuss each of these items (and others) in this text along with instructions on how the develop and compile a “water tight” WWU and Request for Bid sheet.

The Specification Data Sheet

Let’s first look at the Specification Data Sheet. (A closer view can be found on the Specification Data Sheet: EXAMPLE WHICH follows this section. You will also note that this program is in an Excel format and, as customary with Excel, these are columns listed “A” ~ “I”. As you might have already noticed, column “F” is highlighted in orange and “G” through “I” are highlighted in red. There is a reason for this highlighting which will become apparent as we move towards the development of the WWU. Also note that each column has been titled except column “F”: column F is simply a buffer column with no tangible function. OK, let’s look at the content of each column individually.

- Column “A”; “Specification”

This column specifies exactly what needs to be accomplished and the methodology by which it will be installed. It also sets forth the attributes of the material such as thickness, material type, rating, color, finish, etc., and may include a brand name if necessary. This is the most critical component of a good Work Write-Up. It tells the contractor, the home owner, and the grant administrator precisely what will be accomplished and how it will be completed. It also provides each contractor bidding on the project the exact same information. There should be no “wiggle room” in this section.

- Column “B”; “ Spec Number”

This column serves to provide a reference number to a particular specification and allows the writer to use the Excel “search” function to quickly locate the individual specification. When “copying and pasting” a specification from the Specification Data Sheet into the WWU, the Spec # will automatically be entered into the proper cell. Notice that the Spec numbers are sequentially listed but with some numbers omitted between series.

- Column “C”; “Unit of Measure”

This column provides the user with the common “Unit of Measure” appropriate to the material being used. For example: floor covering; carpet, vinyl sheet, and carpet padding are measured in square yards (SY); painting is measured in square foot (SF); trim and molding are measured in linear foot (LF); and roofing is measured in squares (SQ): a “square” is 100 square feet of roof area. There is a full listing of commonly used “Units of Measure” provided in Appendix B. When “copying and pasting” a specification from the Specification Data Sheet into the WWU, the Unit of Measure abbreviation will automatically be entered into the proper cell.

- Column “D”; “Number of Units”

Cells in this column are initially left blank: a number value will be filled in by the Rehab Specialist.

- Column “E”; “Unit Cost”

This column gives a dollar cost per “Unit of Measure”, (e.g. $1.20 per SF, $14.50 per SY, etc.) When “copying and pasting” a specification from the Specification Data Sheet into the WWU, this dollar value will automatically be entered into the proper cell. More about “costs” in a moment.

- Column “F”

This column is intentionally left blank. No value will be entered here.

- Column “G”; “Trade ID”

This column on the Specification Sheet gives a “Trade ID” number. The Trade ID is an arbitrary number assigned to each of the major trade groups.  The significance if the Trade ID number is that it somewhat simplifies your search when seeking that section of the Specification Data Sheet that corresponds to the specific trade you need. See Appendix A for a breakdown of each trade by their “Trade ID” number. Notice that the Trade ID numbers are sequentially listed.  Information in this column will not be transferred on the WWU.

- Column “H”; “Mod Date”

This column gives the latest date the Specification was last written or modified, as well as, the last date on which the “Unit Cost” was sampled. These cells are editable to allow you to update to more current cost per unit if so desired. Information in this column will not be transferred on the WWU.

- Column “I”; “Spec Title”

This column provides a quick title reference to the specification itself. Information in this column will not be transferred on the WWU.

You may modify the contents of column “A, Specification”; column “E, Unit Cost”; and column “H, Mod Date” without any repercussions to the software operation. You are encouraged to make appropriate or necessary modifications to the “Specification” column cells so as to allow compliance to local building codes or community preferences. Be sure, however, to also update the “Unit Cost” and “Mod Date” to reflect these changes. Further, you are welcome to add your own “Specifications”, “Unit Cost”, and “Mod Date” as you so desire.

A word about “Unit Costs” here: most of the “Unit Cost” was derived from sampling done in 1998 and obviously some costs have dramatically changed since then. It’s a good idea to check with local suppliers and/or tradesmen to determine a more accurate reflection of today’s unit cost and make that change in the appropriate cell on the “Specification Data Sheet”.

You may reformat the organization or layout of the Specification Data Sheet: however, prior to performing any modification, it is highly recommended that you perform a “Save As” and provide a new name for this document.  Once done, you may modify the new copy until your heart’s content. However, reformatting may result in formulas contained in specific cells to be compromised and may result in errors in tabulations.

The Work Write-Up form (WWU)

The configuration and layout of the “Work Write-Up Form” (WWU) is designed to compliment the configuration and layout of the Specification Data Sheet: a change in configuration of either the WWU or Specification Data spreadsheet without performing a corresponding change in the other will result in premature baldness. Let’s take a closer look at the configuration and layout of the Work Write-Up form and see how it relates to the Specification Data Sheet.

Note that the column letters and titles on the WWU form do not correspond exactly with the same column letters and titles as they appear on the Specification Data Sheet. For example: column “A” on the Specification Data Sheet is titled “Specification” whereas column “A” on the WWU form is titled “Task #”: “Specification” on the WWU form appears in column “B”. Thereafter, each column letter (“B” ~ “F”) and its corresponding title on the Specification Data Sheet has been shifted one column to the right on the WWU. Column G, “Trade ID”, on the Specification Data Sheet will be titled “Line Item Cost” on the WWU and columns “H” and “I” will not appear on the WWU at all. Incidentally, column “A”, “Task #” on the WWU simply identifies, in numerical sequence, each individual task required to complete the job.

The top of the WWU form will require the Rehab Specialist to fill in certain information specific to the home owner (i.e. name, address, phone #, etc.). Other information concerning the Grant recipient (city or county) and Grant number is required as well. Other “Notes” and “Notices” are set forth in this conspicuous location so as to ensure compliance and good workmanship practices. You can add, delete, or modify these notes and notices as you desire.

The Rehab Specialist must perform a detailed site visit, noting all items needing correcting. He or she will then return to the office with their inspection notes, drawings, measurements, and photographs in hand and will begin to fill out a WWU for the structure. The WWU is a basic and straight forward form. The first task required is to enter the home owners name, address, etc. (As mentioned, those little “Notices” and “Notes” on the front page are there to fully remind the contractor of his scope of work.) Next: enter the “Specification”, “Spec #”, the “Unit of Measure”, the “Number of Units” required, the “Unit Cost”, and the “Line Item Cost” for each item that needs repair or replacement. Total all the “Line Item Cost” at the bottom, add in a percentage for “Profit and Overhead” (if so desired) for the contractor and, voila, you’re done. Not viola huh: OK, let’s make this a bit easier.

With your notes, measurements, drawings, and photos in hand, start with any task you desire. You’ve already determined exactly what needs to be repaired, now select and “copy” (DO NOT “CUT”) the appropriate specification from the Specification Data Sheet then “paste” the data into the rows on the blank WWU form.

Let’s do an example: During the inspection, you have determined that the base molding in bedroom # 1 needs replacing. The bedroom is 10’ x 15’ with a 3’ door opening.

- To start, open a blank WWU form, fill in the entire information specific to the home owner, do a “Save As”, and name the file whatever you like: the name of the homeowner would work fine.

- In column “B” of that new blank row, type in “Bedroom #1”. This signifies that the following task(s) will be performed in Bedroom #1. Continue to insert blank rows between each of the rooms or major work areas. These blank rows serve as a separation between the individual rooms making the WWU easier to read.

- Refer to Appendix A, the “Trade ID” table and find that “trim” installation is a function of the Carpenter trade, Trade ID #10. (Incidentally, the “sub-trades” are listed under the primary trade in the same order as they appear on the Specification Data Sheet.)

- Now open the “Specification Data Sheet”, scroll down column “G, Trade ID” until you find the #10 Trade ID section.

- Scroll through this section, until you find a specification that relates to replacing base molding; in this case it would be “Spec” #2410, “Spec title”: “Baseboard -- 1”x4”.

- Now highlight only columns “A” thru “E” of this row and press “copy”.

- Re-open your WWU form and select only the “B” or “Specification” cell to the right of “Task #1”: press “paste”.

- You will now find that all cells in that row except column “E”, (“No. of Units”) will contain data. Likewise, column “G” (Total line Cost) will contain a formula in a dollar format. Column “E” however, still needs some information concerning the number of units required for the task. Your meticulous notes indicate that the bedroom is 10’ x 15’ with a 3’ door opening. You have determined the length of baseboard to be 47’ (10’+10’+15’+ 15’- 3’ = 47’). Therefore the Rehab Specialist will enter a value of 47 into the appropriate cell in column “E”.

- Upon entering the 47 value, Excel multiplies the “No. of Units”, (47) times the “Unit Cost”, in this case $2.10 per LF, and computes the line item cost to be $98.70 placing that amount in column “G”.

- Repeat this process until all tasks specific to Bedroom #1 have been entered.

- Before listing the tasks to be done in the next room, insert another blank line just below the last row of the Bedroom #1 list. Again, this serves to separate the individual rooms.

You will often find that you need to add additional rows to accommodate your complete WWU. It is better to “copy” a row or two and “paste” them into this section. This allows the formulas to be copied whereas if you simply “insert” a new row, the added row will not contain the appropriate formulas and the continuation of the sequential “Item #’s” will be absent.

When you have completed the WWU, it’s wise to meet with the homeowner, explain each item carefully, and have the homeowner sign and date each page of the WWU. What you as a Rehab Specialist intend to give the homeowner and what the homeowner thinks he is going to get are often worlds apart. This one step will go a long way in preventing conflicts between you, the owner, and the contractor after the work has been completed. This is the reason tight and specific WWU’s are so important.

Tip: to make your WWU more Contractor friendly, group all tasks by name of room (e.g. living room, bedroom #1, bathroom #2, dining room, etc.) or by exterior side (e.g. north exterior wall, south exterior wall, etc.).

After all tasks and values have been entered Excel will sum all of the “Line Item Cost” giving you a “Sub-Total” at the bottom. At this point, you may choose to enter a “Profit and Overhead” percentage that Excel will, in turn, add to the sub-total giving you a final cost estimate. The Profit and Overhead percentage must be determined by the Rehab Advisor based on local costs and customary profit margins. Some locales may be as low as 10~15% while others, especially closer to metropolitan areas, may be as high as 35~40%.

There will, of course, be instances where you will have to modify certain wording in the individual specification on the WWU. Other attributes or descriptive information might need to be added for clarity or to specify a certain brand name, manufacturer, or dealer as you choose.

Finally, “Save” this document in the home owners file. Now you’re ready to create a “Request for Bid” sheet.

The Request for Bid Form (RFB)

The Request for Bid form is much easier to complete as all of the specifications and quantities have already been determined and set forth on the WWU.  The “Request for Bid” form itself is essentially the same as the “Work Write-Up” with a few exceptions:

- The cells in the “Unit Cost” column will be filled in by the bidding contractor: these cells will not contain a dollar value as taken from the Specification Data Sheet or the WWU.

- The cells in the “Total Line Cost” columns will be filled in by the bidding contractor.

- Spaces for the “sub total” and “P&O (profit & overhead) percentage” are not present: these costs are usually added in to each of the Line Item Cost.

- The RFB has two signature lines at the bottom of the page, one for the homeowner the other for the contractor.

Virtually everything else can be “Copied” and “Pasted” from the Work Write-Up form to the “Request for Bid” form.  NOTICE: When copying and pasting data for the WWU be careful delete the dollar value in the “Unit Cost” column of the RFB.  Once you have completed the “Request for Bid” sheet, it’s ready to be reproduced and sent to all potential bidders.

Completing the Work Write-Up and Request for Bid is presented in more detail in the accompanying Tutorial.

The Georgia Department of Community Affairs to make comments, critiques, corrections, and feedback on this program. Please direct these comments to:

Mr. Thomas Spinks

Sr. Housing Consultant

Georgia Department of Community Affairs

60 Executive Park Drive S.E.

Atlanta, Georgia 30329

(404) 679-3128

The Georgia Department of Community Affairs wishes to thank The Enterprise Foundation and the creators and staff of “Housing Developer Pro” for their consent to reproduce the specification library. A full and complete version of “Housing Developer Pro” may be obtained by contacting Mr. Armand Magnelli at (717) 843-4790.